EX NO: 2 Configure VLANs and inter-VLAN routing in Packet Tracer

Objectives:

(i) To create and configure Virtual Local Area Networks (VLANs) for logical network

segmentation and traffic isolation.

(ii) To configure Inter-VLAN Routing to enable communication between devices belonging to

different VLANs.

Software/Apparatus required:

(i) Cisco Packet Tracer

(ii) End Devices (PCs)

(iii) Switches

(iv) Copper Straight-through Cable / Automatic Cable Selector

(v) Routers

Procedure

Objective 1: VLAN Configuration

1. Open Cisco Packet Tracer.

2. Place two switches and six PCs in the workspace.

3. Connect the PCs to the switches using straight-through cables.

4. Connect the two switches using a crossover cable.

5. Create VLAN 10 and VLAN 20 on both switches.

6. Assign the ports connected to PC0, PC1, and PC2 to VLAN 10.

7. Assign the ports connected to PC3, PC4, and PC5 to VLAN 20.

8. Configure the link between the switches as a trunk link.

9. Configure the PCs with the required network settings.

10. Verify communication between devices belonging to the same VLAN.

11. Verify that communication between devices in different VLANs is not possible.

12. Save the configuration and observe the results.

Objective 2: Inter-VLAN configuration

1. Open Cisco Packet Tracer.

2. Place one router, three switches, and six PCs in the workspace.

3. Connect the PCs to their respective switches using straight-through cables.

4. Connect the access switches to the central switch.

5. Connect the router to the central switch.

6. Create VLAN 10 and VLAN 20 on the switches.

7. Assign the ports connected to PC0, PC1, and PC2 to VLAN 10.

8. Assign the ports connected to PC3, PC4, and PC5 to VLAN 20.

9. Configure the links between the switches as trunk links.

10. Configure the router interfaces to route traffic between VLAN 10 and VLAN 20.

11. Configure the required network settings on all PCs.

12. Verify communication between devices within the same VLAN.

13. Verify communication between devices belonging to different VLANs through the

router.

14. Save the configuration and observe the results.

Networking diagram:

Objective 1: VLAN Configuration

Objective 2: Inter-VLAN Configuration

Output:

Objective 1: VLAN Configuration

| SI. No | Source IP | Destination IP | Data sent(bytes) | Data received(bytes) |
| --- | --- | --- | --- | --- |
| 1 | 129.63.10.1 | 129.63.10.2 | 1024 | 1024 |
| 2 | 129.63.10.2 | 129.63.10.3 | 2048 | 2048 |
| 3 | 129.62.20.1 | 129.62.20.2 | 1536 | 1536 |
| 4 | 129.62.20.2 | 129.62.20.3 | 3072 | 3072 |
| 5 | 129.63.10.3 | 129.63.10.1 | 4096 | 4096 |

Objective 2: Inter-VLAN Configuration

| SI. No | Source IP | Destination IP | Data sent(bytes) | Data received(bytes) |
| --- | --- | --- | --- | --- |
| 1 | 192.168.10.1 | 192.168.20.2 | 2048 | 2048 |
| 2 | 192.168.10.2 | 192.168.20.3 | 3072 | 3072 |
| 3 | 192.168.10.3 | 192.168.20.4 | 4096 | 4096 |
| 4 | 192.168.20.2 | 192.168.10.1 | 5120 | 5120 |
| 5 | 192.168.20.4 | 192.168.10.2 | 6144 | 6144 |

Result:

Objective 1: VLAN Configuration

The VLANs were successfully configured, and communication was established only between

devices within the same VLAN while communication between different VLANs was restricted.

Objective 2: Inter-VLAN Configuration

(ii) Inter-VLAN Routing was successfully configured, enabling communication between devices

belonging to different VLANs through the router.