EX NO: 1 Build a LAN topology in Cisco Packet Tracer

Objectives

(i) To implement a star topology using Packet Tracer and hence to transmit data between

the devices connected using a star topology.

(ii) To implement a Mesh topology using Packet Tracer and hence to transmit data

between the devices connected using Mesh topology.

(iii) To implement a hybrid topology using Packet Tracer and hence to transmit data between the

devices connected using a tree topology.

Software/Apparatus required:

(i) Cisco Packet Tracer

(ii) End Devices (PCs)

(iii) Switches

(iv) Copper Straight-through Cable / Automatic Cable Selector

Procedure

Step 1: Start Packet Tracer

 Launch Cisco Packet Tracer.

Step 2: Select Devices

 Choose the required end devices (PCs).

 Select the required networking devices (Switch, Hub, or Bridge) according to the

topology.

Step 3: Build the Network Topology

 Drag and place the required PCs and networking devices into the workspace.

 Arrange the devices according to the specified topology (Star, Mesh, Hybrid, etc.).

Step 4: Connect the Devices

 Select the appropriate cable (Copper Straight-through or Automatic).

 Connect each PC to the corresponding networking device.

 Wait until the interface status changes from amber to green after the STP initialization

process.

Step 5: Configure IP Addressing

 Open each PC.

 Navigate to Desktop, IP Configuration (or Config, FastEthernet0).

 Assign the IP address.

 Configure the subnet mask (or allow Packet Tracer to generate it automatically).

Step 6: Verify Connectivity

 Open Desktop and then Command Prompt.

 Use the ping command to test communication between devices.

 Alternatively, use Add Simple PDU in Realtime or Simulation mode to verify packet

transmission.

 Confirm successful communication through ping replies or successful PDU status.

Networking Diagram:

Objective (i): Implementation of Star topology

Objective (ii): Implementation of Mesh topology

Objective (iii): Implementation of Hybrid Topology

Output

Objective (i): Implementation of Star topology

| SI. No | Source IP | Destination IP | Data sent(bytes) | Data received(bytes) |
| --- | --- | --- | --- | --- |
| 1 | 129.63.32.10 | 129.63.32.11 | 1024 | 1024 |
| 2 | 129.63.32.12 | 129.63.32.13 | 2048 | 2048 |
| 3 | 129.63.32.14 | 129.63.32.15 | 1536 | 1536 |
| 4 | 129.63.32.16 | 129.63.32.17 | 3072 | 3072 |
| 5 | 129.63.32.18 | 129.63.32.10 | 4096 | 4096 |

Objective (ii): Implementation of Mesh topology

| SI. No | Source IP | Destination IP | Data sent(bytes) | Data received(bytes) |
| --- | --- | --- | --- | --- |
| 1 | 129.63.32.10 | 129.63.32.11 | 2048 | 2048 |
| 2 | 129.63.32.11 | 129.63.32.12 | 3072 | 3072 |
| 3 | 129.63.32.12 | 129.63.32.13 | 4096 | 4096 |
| 4 | 129.63.32.13 | 129.63.32.10 | 5120 | 5120 |
| 5 | 129.63.32.10 | 129.63.32.12 | 6144 | 6144 |

Objective (iii): Implementation of Mesh topology

| SI. No | Source IP | Destination IP | Data sent(bytes) | Data received(bytes) |
| --- | --- | --- | --- | --- |
| 1 | 129.63.32.10 | 129.63.32.11 | 2048 | 2048 |
| 2 | 129.63.32.12 | 129.63.32.13 | 3072 | 3072 |
| 3 | 129.63.32.14 | 129.63.32.15 | 4096 | 4096 |
| 4 | 129.63.32.11 | 129.63.32.14 | 5120 | 5120 |
| 5 | 129.63.32.10 | 129.63.32.15 | 6144 | 6144 |

Result

Objective (i): Implementation of Star Topology

The Star topology was successfully created in Cisco Packet Tracer, and communication between

the connected devices was verified successfully.

Objective (ii): Implementation of Mesh Topology

The Mesh topology was successfully simulated in Cisco Packet Tracer, and data transfer between

all connected devices was confirmed.

Objective (iii): Implementation of Hybrid Topology

The Hybrid (Bus–Ring) topology was successfully established in Cisco Packet Tracer, and

successful network connectivity between the connected devices was achieved.